Technical Recommendation

Model：BRTIRUS2030A

Name：Six axis robot

Robot version:21 version

（Without further notice if specification and exterior have been changed because of improvement. Thanks for your understanding.）

- 1. BRTIRUS2030A robot profile

  - 1.1.Introduction of BRTIRUS2030A

BRTIRUS2030A is a six axis robot developed by BORUNTE for complex applications with multiple degrees of freedom. Its maximum loading is 30kg, and it has 2068 mm arm span. The six degrees of freedom flexibility can handle the scenes such as plastic injection part taking, machine tool loading and unloading, assembly, and transportation.

High IP code：Wrist: IP54 Body: IP40

High accuracy：repeated positioning accuracy: ±0.08mm

## 1.3. Application cases of BRTIRUS2030A

(1). injection molding (2). stamping (3). transporting (4). loading and unloading, etc.

- 3. BRTIRUS2030A robot specifications

| Model |  | BRTIRUS2030A |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Axes |  | 6 axis |  |  |  |  |  |  |
| Loading ability |  | 30kg |  |  |  |  |  |  |
| IP code |  | Body: IP40 Wrist: IP54 |  |  |  |  |  |  |
| Repeated positioning accuracy |  | ±0.05mm |  |  |  |  |  |  |
| Maximum arm span |  | 2068mm |  |  |  |  |  |  |
| Weight |  | 310kg |  |  |  |  |  |  |
| Max speed | J1 | 102°/s |  | J2 |  | 103°/s |  |  |
|  | J3 | 123°/s |  | J4 |  | 245°/s |  |  |
|  | J5 | 270°/s |  | J6 |  | 337°/s |  |  |
| Motion range | J1 | ±150° |  | J2 |  | -90°/+70° |  |  |
|  | J3 | -55°/+105° |  | J4 |  | ±180° |  |  |
|  | J5 | ±115° |  | J6 |  | ±360° |  |  |
| Maximum rotational torque | J4 | 54.1N*m | J5 | 67.4N*m |  | J6 |  | 60.7N*m |
| Maximum moment of inertia | J4 | 1.69kg*m2 | J5 | 1.54kg*m2 |  | J6 |  | 1.5kg*m2 |
| Mounting environment | Operating temperature： 0℃~40℃ Best temperature：25℃~40℃ Relative humidity：20~80%RH（No condensation） |  |  |  | Mpa |  | 0.5~0.7MPa |  |
|  |  |  |  |  | Power supply |  | 220V±10％ 50Hz/60Hz |  |
| Core components | Name | Operating system |  | Brand |  | BORUNTE |  |  |
|  |  | Casting body |  |  |  | BORUNTE |  |  |
|  |  | Servo motor |  |  |  | BORUNTE |  |  |
|  |  | Reducer |  |  |  | BORUNTE |  |  |
|  |  | Electrical component |  |  |  | BORUNTE |  |  |
| Oil content | J1 | 2200ML | J2 | 1100ML |  | J3 |  | 1100ML |
|  | J4 | 700ML | J5 | / |  | J6 |  | / |
| Vulnerable parts | Timing belt | MBT233090502 Timing belt 450-5MHP-9 bandwidth - 90 teeth |  |  |  |  |  |  |
| Robot accompanying materials |  | USB |  |  |  |  |  |  |

The rated power, specifications, external dimensions, etc. of this product are subject to modification without prior notice.

Technical data and illustrations are for supply reference only and reserve the right to make changes.